List 1: Hazardous Wastes to be controlled in Accordance with Schedule I Environmental Protection (Hazardous Wastes Management) Regulations, 2000

Waste streams

- Clinical Wastes from medical care in hospitals, medical centers and clinics.

- Wastes from the production and preparation of pharmaceutical products.

- Wastes from the production, formulation and use of biocides and phytopharmaceuticals.

- Waste pharmaceuticals, drugs and medicines.

- Wastes from the manufacture, formulation and use of wood preserving chemicals.

Waste having as Constituents

- Metal carbonyls

- Beryllium, beryllium compounds

- Hexavalent chromium compounds

- Copper compounds

- Zinc compounds

- Arsenic, arsenic compounds

- Selenium, selenium compounds

- Cadmium, cadmium compounds

- Antimony, antimony compounds

- Tellurium, tellurium compounds

- Mercury, mercury compounds

- Thallium, thallium compounds

- Lead, lead compounds

- Inorganic fluoride compounds excluding calcium fluoride

- Inorganic cyanides

- Acidic solutions or acids in solid form

- Basic solutions or bases in solid form

- Asbestos (dust and fibres)\Organic phosphorous compounds

- Organic cyanides

- Phenols, phenol compounds including chlorophenols

- Ethers

- Halogenated organic solvents

- Any congenor of polychlorinated bibenzo-furan

- Any congenor of polychlorinated dibenzo-p-dioxin

- Organohalogen compounds other than substances referred to in this Schedule.

List 2: Hazardous Characteristics in Accordance with Schedule II Environmental Protection (Hazardous Wastes Management) Regulations 2000

- Explosives.

- Flammable liquids.

- Flammable solids or waste solids other than explosives which may be readily combustible.

- Oxidizing (corroding) substances.

- Organic peroxides.

- Poisonous substances.

- Infectious substances.

- Corrosives.

- Toxic gases.

- Toxic substances which if inhaled or ingested may cause delayed or chronic effects.

- Ecotoxic substances which if released may present immediate or delayed adverse impacts to the environment by means of bioaccumulation and/or toxic effects upon systems.

- Materials capable, after disposal, of yielding another material which possesses any of the characteristics specified in items 1 – 11.

TABLE: Types of Hazardous Wastes, Waste Streams and Characteristics

| Hazardous Waste Stream | Hazardous Waste/Materials | Hazardous Constituents | Hazardous Characteristics |
| --- | --- | --- | --- |
| Clinical waste (from hospitals, medical centres and clinics) | Blood, body fluids Sharp : syringes , needles, etc Laboratory waste: stock culture Human and animal tissues Dressing : surgical dressing, swabs, surgical gloves | Same as actual materials Beryllium from radiation windows for X-ray tubes, Formalin, Stock culture infectious agents, Unsterilized material used in stock culture | Infectious Toxic |
| Wastes from the production and preparation of pharmaceutical products | Strong acid and Bases Preservatives, Spent catalyst | Solvents & Organic acids, Tellurium, Organochlorides, Organophosphates, Waste water may contain Mercury, Cadmium, 1,2-dichloroethane, Particulate matter Volatile organic compounds. Liquids effluent from cleaning and after batch operation | Toxic, Poisonous Corrosives |
| Wastes from production, formulation and use of biocides and phytopharmaceuticals | Biocide: Oxidizing agents Non-oxidizing agents | Chlorine Chlorine dioxide Chloroisocyanurates Hypochlorite Acrolein Amines Chlorinated phenolics Copper salts Organo-sulphur compounds Quaternary ammonium salts | Toxic Poisonous |
| Hazardous Waste Stream | Hazardous Waste/Materials | Hazardous Constituents | Hazardous Characteristics |
| Waste pharmaceutical drugs and medicine | Strong acid and bases Preservatives Chemotherapy drugs, tracers/ markers Other drugs, Antimicrobacterials, Hormones & Endocrine disrupting drugs. | Glacial acetic acid , Concentrated sodium hydroxide, Nitroglycerine Dichlorodifluoromethane Hexachlorophene, Arsenic, Barium, Cadmium, Chloroform, Chromium, Erythromycin gel 2%, Texacort solution 1%, Taxol injection, | Toxic Ignitable Corrosive Radioactive |
| Waste from the manufacture , formulation, and use of wood preserving chemicals | Water –borne preservatives Toxic Oil-borne preservatives Light organic solvent preservatives Borate preservatives | Copper azole, Chromate copper arsenate Alkaline copper quaternary Sodium silicate based Potassium silicate based Pentachlorophenol Coal -tar Creosote Linseed oil White spirit Kerosene Boric acid Oxides | Toxic |

| Hazardous Waste/ Material Constituents | Actual Substances/ Uses | Hazardous Characteristics |
| --- | --- | --- |
| Metal Carbonyls | Used in the production of carbon steel, used as catalyst in organic reactions | Toxic Flammable |
| Beryllium, Beryllium Compounds | Used mostly in military application, Radiation windows for X-ray tubes, Computers, Televisions, Aerospace Industry, Aircrafts, and Communication Satellites. | Toxic |
| Hexavalent Chromium Compounds | Stainless Steel, Leather tanning, Anti- Corrosion, wood preservatives | Toxic Oxidizing |
| Copper Compounds | 98% of metal is used in Electrical Devices, Architecture Industry, Released from sewage treatment processes, Phosphate Fertilizer Production | Eco – Toxic |
| Zinc Compounds | Galvanizing, Brass, Anti- corrosion , Batteries | Toxic Eco- Toxic |
| Arsenic, Arsenic Compounds | Pesticides, Herbicides and Insecticides | Toxic Eco- Toxic |
| Selenium, Selenium Compounds | Used in electronics (photocells, solar cells), used in glass industry, used in toning of photographs | Toxic |
| Cadmium, Cadmium Compounds | Batteries, Paint pigments , In PVC as a weathering, light and heat stabilizer | Toxic Eco- Toxic |
| Antimony, Antimony Compounds | Fire retardants, Paints , Lead storing Batteries | Toxic Eco- Toxic |
| Tellurium, Tellurium Compounds | Used in Lead, Copper & Iron alloys, Used to colour ceramics | Toxic |
| Mercury, Mercury Compounds | Batteries ,Thermometers, Disinfectants, Photocopy Tonners, Silver & Gold Mining Industry, Used in Cosmetics, | Toxic Eco- Toxic |
| Thallium, Thallium compounds | Rat poisons, Ant killers, Electronic Industries, Optical glasses. | Toxic |
| Lead, Lead Compounds | Batteries, Cable covering, Ceramic glazes, sheet lead. | Toxic Eco- Toxic |
| Hazardous Waste/ Material Constituents | Actual Substances/ Uses | Hazardous Characteristics |
| Inorganic fluoride compounds excluding Calcium Fluoride ( e.g. Hydrogen Fluoride) | Pesticides, Tooth paste, Insecticides | Could produce Toxic Gases when burnt Eco- Toxic |
| Inorganic cyanides (e.g. Barium Cyanide, Hydrogen Cyanide, Lead Cyanide) | Smoke from burning plastic containing Nitrogen, Used in Electroplating, photographic development, manufacture of fumigation chemicals, Insecticides | Toxic Gas Toxic |
| Acidic solutions or acids in solid form (e.g. Hydrochloric Acid) | Used in the production of Vinyl Chloride for PVC, pickling of steel (rust removal), Leather processing | Corrosives |
| Basic solutions or bases in solid forms( e.g. Calcium Carbonate, Sodium hydroxide, potassium hydroxide) | Used in manufacturing of pulp& paper, Detergents, Drain cleaner, biodiesel production, found in cement | Toxic |
| Asbestos (dust and fibres) Organic Phosphorous Compounds | Asphalt floor tiles, Vinyl sheet flooring, Fire proofing material, Roofing shingles, chalk board | Toxic |
| Organic Cyanides ( Nitriles) | Benonitrile, Acrylonitrile, Ethanenitrile | Toxic |
| Phenols, phenol compounds including chlorophenols | Pharmaceuticals, Herbicides | Toxic |
| Ether | Used in Solvents of Resins, oils, fats and Gum, Used as a fuel, Paints, Dyes, used in Refrigeration | Toxic Flammable Liquids |
| Halogenated Organic Solvents | Dry cleaning products, Thermoplastic production, Degreasing , Paints | Toxic |
| Any congener of polychlorinated dibenzo- furan | Formed by paralysis or incineration from producing chlorine containing products such as PVCs and PCBs | Toxic |
| Any congener of polychlorinated dibenzo-p-dioxin ( AKA Dioxins) | They occur as by products of from the manufacture of organochlorides, incineration of chlorine containing substances such as PVC and bleaching of paper. | Toxic Eco- Toxic |
| Organ halogen compounds other than those mentioned in table | Fire extinguishers, Fumigants , Solvents in rubber polymerization, Production of pharmaceuticals | Toxic Substances |